磁流体密封装置使用说明

1．灰尘和溶剂（水、油、酒精、丙酮、氟利昂等）不能侵入磁流体密封装置；

2．泄漏检查使用氦质谱检漏仪进行；

3．不得加上超出规定的压力，真空场合时加压1kg/cm2，加压场合时，只能加压到技术说明规定的压力为止；

4．磁流体密封装置的转轴在静止时，对真空腔抽真空或真空中长时间停止后再运转时，会导致真空压力上升，这个是磁流体密封装置的固有现象，可以通过提早起动等措施来消除；

5．水冷产品使用时，需使用常温、压力为1-3kg/cm2的水，冷却水可以使用普通的自来水(氯离子200mg/L，铁离子0.3mg/L)。此外，装置内不能结水，因为这样会导致轴承生锈，从而使回转不良。

Vacuum Feedthroughs Operation Instructions

- Prevent dust and solvents (water, oil, alcohol, acetone, Freon, etc) contaminating the vacuum feedthroughs.

- Using Helium mass spectrum leak detector to detect the leakage rates.

- It can not be loaded too much pressure beyond its maximal permitted pressure. At the vacuum occasion, it is permitted to load 1 kg/cm2; at the pressurizing occasion, it is only permitted to pressurize within the range of technology allowing.

- If vacuumizing the vacuum-chamber when the shaft of vacuum feedthrough is at rest or the vacuum feedthruogh begins to work after long-time stopping in vacuum, it may cause a rise in vacuum pressure. This is a connatural phenomenon of vacuum feedthroughs, which can be avoided through advanced startup.

- When it needs water cooling, the cooling water should be of normal temperature and 1-3 kg/cm2 pressure, which could be ordinary tap water (chlorine ion 200 mg/L, iron ion 0.3mg/L). Besides, water can not exist in the inner parts of vacuum feedthroughs which may cause rust of bearing and circumrotating failure.